ADS NITRO

# Product Technical Specification:  ADS NITRO

# :

# Product Technical Specification:  ADS NITRO

# :

| Specification |  |
| --- | --- |
| Available Nozzles | 1 |
| Nozzle Reach | 10 m Hook (Perpetual) (Hose Reel Optional) |
| Display Options | - 3 line 1.25-inch display for transaction data - Graphic Display for Configurations, Errors, messages and other explicit data. - Multi-functional 23 key keypad - Smart indication system - Segment LCD with Backlight display - Hardware lock for calibration - Secure passwords for calibration & Multiple Dispensing Mode |
| Working temperature | (0 to + 55°C) |
| Relative Humidity | 5 to 95% |
| Voltages | 180 V to 300V AC Single Phase; 50/60 Hz AC |
| Rated Flow Rate Practical Flow Rate | 30 LPM 20 LPM |
| Pre-set | Pre-Set (from ERP or local) & Manual (Volumetric and Amount based) |
| Flow Meter Accuracy | 0.2% |
| Hose Length | 4 Meters Nozzle Hose pipe |
| Enclosure | MS Powder Coated heavy ENCLOSURE with thermal insulation of 30mm (For better Rust Protection) |
| Boot-up | Boot-Up Log (For identifying power loss and frequent restart conditions) |
| Transaction | Shift settlement and Records for easy monitoring of financial transactions. |
| No-Cheating | Reset to Zero announcements over speaker for drivers (FOR no-Cheating while dispensing) |
| Storage | 3 months or 15000 transactions local Storage Available |
| Relay | Power Control Relay for Electronics Input Power control. |
| Single Nozzle | Auto cut-off/ Pressure Sensitive with latch, colours cuff guard |
| Breakaway | Nozzle mounted |
| Connectivity | IOT enabled Communication options include Wi-Fi / Ethernet / RS485 / RS232 |
| Totalizer | Electronic Non-Resettable Totalizer |
| Printer | - Receipt Printer, Inbuilt 2-inch Thermal Printer for Receipt Printing, Paper/Printing width-58/48mm - Paper roll dia./Length-40mm/18meter - Communication protocol-RS232 - Power Supply: 5V, 2.5 A - High Resolution Printing Low Power Consumption |
| Security | Enhanced Security Features |
|  | AES 256 Bit Encryption |
|  | One-Time password for calibration & replacement of critical components |
|  | Rate control can be change from LOCAL to Central and vice Versa Remote locking of Dispenser for unauthorized Usage. |
|  | Non-resettable electronics totalizer |
| ERP & Management | Integration With Client ERP using CSV data over TCPIP for easy Integration. SET ACCESS RIGHTS and limitations for Different Employee categories (Attendant / Accountant / Manager) |
| Structure | Comparatively Integrated, small and Robust design |
| GSM | - Enables real-time data transmission and - monitoring via 2G/4G networks - Supports OTA (Over-the-Air) firmware - updates for remote maintenance - Compatible with mobile app for dispenser - configuration and diagnostics - Built-in data buffer ensures offline data is - auto-synced when network is restored |
| Features | - Dispenser Registration - Dispenser configuration - Transactions Logs - Shift Log (3 Shift/Day) - Error logs - Calibration Logs - Configuration Logs - Accessible Multiple pump managers - Real time data monitoring - Firmware updates through OTA - Data Recovery |

Commercial Offer for ADS NITRO (DEF)

| Sr. No. | Description | Total Order Quantity | Minimum Order Quantity in a Lot | Ex Work Price Per Pcs (Excluding taxes & transit cost) |
| --- | --- | --- | --- | --- |
| 1 | Advanced DEF Dispenser Set. | 20 | 5 | 72,000.00 INR |